# 1. Topología del Proyecto

El siguiente diagrama representa la topología lógica completa de la red empresarial de EMPRESA DEL GRUPO MORATTI, con tres sedes interconectadas mediante túneles GRE/IPSec sobre Internet.

Figura 1 — Topología lógica de la red empresarial multisede

## Resumen de la Topología

La red está diseñada bajo una arquitectura jerárquica de tres capas (Core, Distribución y Acceso) distribuida en tres sedes geográficas:

| Sede | Función | OSPF Área | Red LAN |
| --- | --- | --- | --- |
| Santo Domingo (Central) | Sede principal, Core, distribución con HSRP, NAT/VPN Hub | Área 1 | 192.168.10–40.0/25 |
| Santiago | Sede sucursal, Data Center, Ventas, Admin | Área 2 | 192.168.50.0/x |
| La Romana | Sede sucursal remota, tres departamentos | Área 3 | 192.168.60.0/x |
| Puerto Plata | Sede sucursal remota, tres departamentos con crecimiento x1.4 | Área 4 | 192.168.70.0/x |
| Barahona | Sede sucursal remota, tres departamentos con crecimiento x1.4 | Área 5 | 192.168.80.0/x |
| Backbone VPN | Túneles GRE/IPSec entre los cinco routers WAN | Área 0 | 192.168.200.0/30 |

Las sedes están interconectadas mediante túneles GRE protegidos con IPSec (AES-256 / SHA-256 / DH Group 14), formando el Área 0 de OSPF (Backbone VPN). Cada sede tiene su propia área OSPF:

| Router | Router-ID | Áreas OSPF | Loopback 0 | Rol |
| --- | --- | --- | --- | --- |
| R-CORE | 1.1.1.1 | Área 0 (VPN), Área 1 | 1.1.1.1/32 | ABR / Hub VPN |
| R-SW1 | 11.11.11.1 | Área 1 | 11.11.11.1/32 | Distrib. SD (HSRP) |
| R-SW2 | 22.22.22.1 | Área 1 | 22.22.22.1/32 | Distrib. SD (HSRP) |
| R-SANTIAGO | 2.2.2.2 | Área 0 (VPN), Área 2 | 2.2.2.2/32 | ABR / Gateway STG |
| R-ROMANA | 3.3.3.3 | Área 0 (VPN), Área 3 | 3.3.3.3/32 | ABR / Gateway ROM |
| R-PUERTOPLATA | 4.4.4.4 | Área 0 (VPN), Área 4 | 4.4.4.4/32 | ABR / Gateway PP |
| R-BARAHONA | 5.5.5.5 | Área 0 (VPN), Área 5 | 5.5.5.5/32 | ABR / Gateway BAR |
| FSP | 10.0.0.1 | N/A | 10.0.0.1/32 | Simulación ISP |

# 2. Tabla de Direccionamiento

La siguiente tabla consolida la asignación de direcciones IP tanto privadas como públicas para todos los dispositivos y segmentos de la red empresarial.

## 2.1 Sede Central — Santo Domingo (OSPF Área 1)

| VLAN | Nombre | Red | Gateway (HSRP) | Hosts | Máscara |
| --- | --- | --- | --- | --- | --- |
| 10 | DEPTO1 | 192.168.10.0/24 | 192.168.10.1 | 110 (×1.4→154) | /24 |
| 20 | DEPTO2 | 192.168.20.0/25 | 192.168.20.1 | 51 (×1.4→71) | /25 |
| 30 | DEPTO3 | 192.168.30.0/25 | 192.168.30.1 | 64 (×1.4→90) | /25 |
| 40 | DEPTO4 | 192.168.40.0/25 | 192.168.40.1 | 80 (×1.4→112) | /25 |
| 99 | MANAGEMENT | 192.168.99.0/24 | 192.168.99.1 (HSRP) | — | /24 |
| 999 | NATIVA | — | — | — | — |

## 2.2 Sede Santiago (OSPF Área 2)

| VLAN | Nombre | Red | Gateway | Hosts | Máscara |
| --- | --- | --- | --- | --- | --- |
| 10 | Centro de Datos | 192.168.50.0/28 | 192.168.50.1 | 10 | /28 |
| 20 | Ventas | 192.168.50.32/27 | 192.168.50.33 | 15 | /27 |
| 30 | Administración | 192.168.50.64/28 | 192.168.50.65 | 5 | /28 |
| 99 | MANAGEMENT | 192.168.99.20/24 | 192.168.99.20 | — | /24 |
| 999 | NATIVA | — | — | — | — |

## 2.3 Sede La Romana (OSPF Área 3)

| VLAN | Nombre | Red | Gateway | Hosts | Máscara |
| --- | --- | --- | --- | --- | --- |
| 10 | DEPTO1-ROM | 192.168.60.0/26 | 192.168.60.1 | 25 | /26 |
| 20 | DEPTO2-ROM | 192.168.60.128/25 | 192.168.60.129 | 52 | /25 |
| 30 | DEPTO3-ROM | 192.168.60.64/28 | 192.168.60.65 | 7 | /28 |
| 99 | MANAGEMENT | 192.168.99.30/24 | 192.168.99.30 | — | /24 |
| 999 | NATIVA | — | — | — | — |

## 2.4 Sede Puerto Plata (OSPF Área 4)

| VLAN | Nombre | Red | Gateway | Hosts | Máscara |
| --- | --- | --- | --- | --- | --- |
| 10 | DEPTO1-PP | 192.168.70.0/26 | 192.168.70.1 | 25→35 | /26 |
| 20 | DEPTO2-PP | 192.168.70.64/25 | 192.168.70.65 | 52→73 | /25 |
| 30 | DEPTO3-PP | 192.168.70.192/28 | 192.168.70.193 | 7→10 | /28 |
| 99 | MANAGEMENT | 192.168.99.40/24 | 192.168.99.40 | — | /24 |
| 999 | NATIVA | — | — | — | — |

## 2.5 Sede Barahona (OSPF Área 5)

| VLAN | Nombre | Red | Gateway | Hosts | Máscara |
| --- | --- | --- | --- | --- | --- |
| 10 | DEPTO1-BAR | 192.168.80.0/26 | 192.168.80.1 | 25→35 | /26 |
| 20 | DEPTO2-BAR | 192.168.80.64/25 | 192.168.80.65 | 52→73 | /25 |
| 30 | DEPTO3-BAR | 192.168.80.192/28 | 192.168.80.193 | 7→10 | /28 |
| 99 | MANAGEMENT | 192.168.99.50/24 | 192.168.99.50 | — | /24 |
| 999 | NATIVA | — | — | — | — |

## 2.6 Servidores — Centro de Datos Santiago

| IP | Rol / Servicio | VLAN | Switch |
| --- | --- | --- | --- |
| 192.168.50.2 | DNS / WEB / DHCP | 10 | SW-STGO1 (e0/1) |
| 192.168.50.3 | FTP / RADIUS | 10 | SW-STGO1 (e0/2) |
| 192.168.50.4 | MAIL | 10 | SW-STGO1 (e0/3) |
| 192.168.1.2 | Servidor Web SD (público) | — | R-CORE (e0/3) |

## 2.7 Enlaces WAN Públicos (19.0.0.0/24)

| Enlace | IP Extremo A | IP Extremo B | Máscara | Red |
| --- | --- | --- | --- | --- |
| R-CORE ↔ FSP | 19.0.0.1 (R-CORE e0/0) | 19.0.0.2 (FSP e0/0) | /30 | 19.0.0.0/30 |
| FSP ↔ R-SANTIAGO | 19.0.0.5 (FSP e0/1) | 19.0.0.6 (R-SANTIAGO e0/0) | /30 | 19.0.0.4/30 |
| FSP ↔ R-ROMANA | 19.0.0.9 (FSP e0/2) | 19.0.0.10 (R-ROMANA e0/0) | /30 | 19.0.0.8/30 |
| FSP ↔ R-PUERTOPLATA | 19.0.0.13 (FSP e0/3) | 19.0.0.14 (R-PUERTOPLATA e0/0) | /30 | 19.0.0.12/30 |
| FSP ↔ R-BARAHONA | 19.0.0.17 (FSP e1/0) | 19.0.0.18 (R-BARAHONA e0/0) | /30 | 19.0.0.16/30 |

## 2.8 Túneles GRE/IPSec — Backbone OSPF Área 0

| Interfaz | Enlace | IP Local | IP Remota | Área OSPF |
| --- | --- | --- | --- | --- |
| Tunnel0 (R-CORE) | R-CORE → R-SANTIAGO | 192.168.200.1/30 | 192.168.200.2/30 | Área 0 |
| Tunnel1 (R-CORE) | R-CORE → R-ROMANA | 192.168.200.5/30 | 192.168.200.6/30 | Área 0 |
| Tunnel0 (R-SANTIAGO) | R-SANTIAGO → R-CORE | 192.168.200.2/30 | 192.168.200.1/30 | Área 0 |
| Tunnel0 (R-ROMANA) | R-ROMANA → R-CORE | 192.168.200.6/30 | 192.168.200.5/30 | Área 0 |
| Tunnel2 (R-CORE) | R-CORE → R-PUERTOPLATA | 192.168.200.9/30 | 192.168.200.10/30 | Área 0 |
| Tunnel3 (R-CORE) | R-CORE → R-BARAHONA | 192.168.200.13/30 | 192.168.200.14/30 | Área 0 |
| Tunnel0 (R-PUERTOPLATA) | R-PUERTOPLATA → R-CORE | 192.168.200.10/30 | 192.168.200.9/30 | Área 0 |
| Tunnel0 (R-BARAHONA) | R-BARAHONA → R-CORE | 192.168.200.14/30 | 192.168.200.13/30 | Área 0 |

## 2.9 Interfaces de Gestión (VLAN 99)

| Dispositivo | IP Gestión (VLAN 99) | Gateway por defecto | Tipo |
| --- | --- | --- | --- |
| R-SW1 | 192.168.99.2/24 | 192.168.99.1 (HSRP) | Multilayer Switch |
| R-SW2 | 192.168.99.3/24 | 192.168.99.1 (HSRP) | Multilayer Switch |
| SW1 (Depto 1 SD) | 192.168.99.11/24 | 192.168.99.1 | Access Switch |
| SW2 (Depto 2 SD) | 192.168.99.12/24 | 192.168.99.1 | Access Switch |
| SW3 (Depto 3 SD) | 192.168.99.13/24 | 192.168.99.1 | Access Switch |
| SW4 (Depto 4 SD) | 192.168.99.14/24 | 192.168.99.1 | Access Switch |
| R-SANTIAGO | 192.168.99.20/24 | — | Router/Gateway |
| SW-STGO1 | 192.168.99.21/24 | 192.168.99.20 | Access Switch |
| SW-STGO2 | 192.168.99.22/24 | 192.168.99.20 | Access Switch |
| R-ROMANA | 192.168.99.30/24 | — | Router/Gateway |
| SW-ROM | 192.168.99.31/24 | 192.168.99.30 | Access Switch |
| R-PUERTOPLATA | 192.168.99.40/24 | — | Router/Gateway |
| SW-PP | 192.168.99.41/24 | 192.168.99.40 | Access Switch |
| R-BARAHONA | 192.168.99.50/24 | — | Router/Gateway |
| SW-BAR | 192.168.99.51/24 | 192.168.99.50 | Access Switch |

# 3. Tabla de VLANs

El esquema de VLANs es consistente en todas las sedes para simplificar la administración y facilitar el trunking entre routers y switches.

| ID VLAN | Nombre | Propósito | Sedes |
| --- | --- | --- | --- |
| 10 | DEPTO1 / CENTRO-DATOS | Departamento 1 (SD) / Centro de Datos (STG) / Depto 1 (ROM) | SD / STG / ROM / PP / BAR |
| 20 | DEPTO2 / VENTAS | Departamento 2 (SD) / Ventas (STG) / Depto 2 (ROM) | SD / STG / ROM / PP / BAR |
| 30 | DEPTO3 / ADMINISTRACION | Departamento 3 (SD) / Administración (STG) / Depto 3 (ROM) | SD / STG / ROM / PP / BAR |
| 40 | DEPTO4 | Departamento 4 (exclusivo Sede Santo Domingo) | SD |
| 99 | MANAGEMENT | Gestión out-of-band de todos los dispositivos de red | SD / STG / ROM / PP / BAR |
| 999 | NATIVA | VLAN nativa en todos los trunks (sin tráfico de datos) | SD / STG / ROM / PP / BAR |

# 4. Configuración de Dispositivos

A continuación se presenta la configuración completa de cada dispositivo intermedio de la red, organizado por sede y función. Todos los dispositivos comparten la siguiente política de seguridad base:

• Enable secret: MORATTI2025

• Usuario admin: Admin@ITLA2025 (privilege 15)

• SSH versión 2 con clave RSA 2048 bits

• Dominio: moratti.com.do

• Acceso VTY restringido a VLAN de Gestión (192.168.99.0/24)

## 4.1  FSP — Router ISP Simulado

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Router (Simulación ISP) |
| Ubicación | Nube / Internet |
| Descripción | Simula el proveedor de servicios de Internet. Conecta las tres sedes a través de la red pública 19.0.0.0/24. No participa en OSPF. |

Configuración completa:

hostname FSP

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

username admin privilege 15 secret Admin@ITLA2025

ip domain-name moratti.com.do

crypto key generate rsa modulus 2048

ip ssh version 2

banner motd ^

*** ACCESO RESTRINGIDO — EMPRESA MORATTI ***

Solo personal autorizado. Toda actividad es monitoreada.

^

interface Loopback0

ip address 10.0.0.1 255.255.255.255

interface e0/0

description "Enlace a R-CORE (Santo Domingo)"

ip address 19.0.0.2 255.255.255.252

no shutdown

interface e0/1

description "Enlace a R-SANTIAGO (Santiago)"

ip address 19.0.0.5 255.255.255.252

no shutdown

interface e0/2

description "Enlace a R-ROMANA (La Romana)"

ip address 19.0.0.9 255.255.255.252

no shutdown

ip route 19.0.0.0 255.255.255.0 Null0

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

end

## 4.2  R-CORE — Router Sede Central Santo Domingo

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Router Cisco (con soporte IPSec/GRE) |
| Ubicación | Sede Santo Domingo — Capa Core |
| Descripción | Router principal de la empresa. Actúa como Hub VPN hacia Santiago y La Romana. Implementa NAT overload, IPSec/GRE, OSPF multi-área (ABR) y es la puerta de enlace hacia Internet para toda la sede SD. |

Configuración completa:

hostname R-CORE

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

ip routing

username admin privilege 15 secret Admin@ITLA2025

ip domain-name moratti.com.do

crypto key generate rsa modulus 2048

ip ssh version 2

ISAKMP POLICY (IKE Fase 1) ---

crypto isakmp policy 10

encr aes 256

hash sha256

authentication pre-share

group 14

lifetime 86400

crypto isakmp key Moratti@2025 address 19.0.0.6

crypto isakmp key Moratti@2025 address 19.0.0.10

IPSEC TRANSFORM SET (IKE Fase 2) ---

crypto ipsec transform-set TSET esp-aes 256 esp-sha256-hmac

mode transport

crypto ipsec profile VPN-PROFILE

set transform-set TSET

INTERFACES ---

interface Loopback0

description "Router-ID OSPF"

ip address 1.1.1.1 255.255.255.255

interface e0/0

description "WAN — Enlace a FSP"

ip address 19.0.0.1 255.255.255.252

ip nat outside

ip access-group PROTECT-WAN in

no shutdown

interface e0/1

description "LAN — Enlace a R-SW1"

ip address 192.168.100.1 255.255.255.252

ip nat inside

ip ospf 763 area 1

no shutdown

interface e0/2

description "LAN — Enlace a R-SW2"

ip address 192.168.100.5 255.255.255.252

ip nat inside

ip ospf 763 area 1

no shutdown

interface e0/3

description "LAN — Servidor Web SD"

ip address 192.168.150.1 255.255.255.248

ip nat inside

ip ospf 763 area 1

no shutdown

TUNEL GRE/IPSEC hacia Santiago ---

interface Tunnel0

description "VPN-GRE → R-SANTIAGO"

ip address 192.168.200.1 255.255.255.252

ip ospf 763 area 0

tunnel source e0/0

tunnel destination 19.0.0.6

tunnel protection ipsec profile VPN-PROFILE

no shutdown

TUNEL GRE/IPSEC hacia La Romana ---

interface Tunnel1

description "VPN-GRE → R-ROMANA"

ip address 192.168.200.5 255.255.255.252

ip ospf 763 area 0

tunnel source e0/0

tunnel destination 19.0.0.10

tunnel protection ipsec profile VPN-PROFILE

no shutdown

OSPF ---

router ospf 763

router-id 1.1.1.1

passive-interface e0/0

passive-interface e0/3

default-information originate

NAT OVERLOAD ---

ip access-list extended NAT-ACL-SD

deny   ip 192.168.0.0 0.0.255.255 192.168.0.0 0.0.255.255

permit ip 192.168.0.0 0.0.255.255 any

ip nat inside source list NAT-ACL-SD interface e0/0 overload

ACL WAN PROTECTION ---

ip access-list extended PROTECT-WAN

deny   tcp any any eq 23

deny   tcp any any eq 513

deny   tcp any any eq 514

permit ip any any

RUTA POR DEFECTO ---

ip route 0.0.0.0 0.0.0.0 19.0.0.2

SSH/VTY ---

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.3  R-SW1 — Multilayer Switch Distribución SD

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Catalyst Multilayer Switch (L3) |
| Ubicación | Sede Santo Domingo — Capa Distribución |
| Descripción | Switch de distribución principal en SD. Funciona como router de capa 3 para las VLANs 10, 30, 99 (HSRP Activo) y 20, 40 (HSRP Standby). Implementa DHCP, OSPF área 1, STP Rapid-PVST, DHCP Snooping y DAI. |

Configuración completa:

hostname R-SW1

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

ip routing

VLANs ---

vlan 10

name DEPTO1

vlan 20

name DEPTO2

vlan 30

name DEPTO3

vlan 40

name DEPTO4

vlan 99

name MANAGEMENT

vlan 999

name NATIVA

UPLINK A R-CORE ---

interface e0/0

no switchport

ip address 192.168.100.2 255.255.255.252

ip ospf 763 area 1

no shutdown

TRUNK A R-SW2 (redundancia) ---

interface e0/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

no shutdown

TRUNK A SW1 (VLAN 10) ---

interface e0/1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

no shutdown

TRUNK A SW2 (VLAN 20) ---

interface e0/2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 20,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

no shutdown

SVIs CON HSRP ---

interface vlan 10

ip address 192.168.10.2 255.255.255.0

standby version 2

standby 10 ip 192.168.10.1

standby 10 priority 110

standby 10 preempt delay minimum 30

standby 10 timers 1 3

ip ospf 763 area 1

no shutdown

interface vlan 20

ip address 192.168.20.2 255.255.255.128

standby version 2

standby 20 ip 192.168.20.1

standby 20 priority 90

standby 20 preempt delay minimum 30

standby 20 timers 1 3

ip ospf 763 area 1

no shutdown

interface vlan 30

ip address 192.168.30.2 255.255.255.128

standby version 2

standby 30 ip 192.168.30.1

standby 30 priority 110

standby 30 preempt delay minimum 30

ip ospf 763 area 1

no shutdown

interface vlan 40

ip address 192.168.40.2 255.255.255.128

standby version 2

standby 40 ip 192.168.40.1

standby 40 priority 90

standby 40 preempt delay minimum 30

ip ospf 763 area 1

no shutdown

interface vlan 99

ip address 192.168.99.2 255.255.255.0

standby version 2

standby 99 ip 192.168.99.1

standby 99 priority 110

standby 99 preempt delay minimum 30

ip ospf 763 area 1

no shutdown

DHCP POOLS ---

ip dhcp excluded-address 192.168.10.1  192.168.10.10

ip dhcp excluded-address 192.168.20.1  192.168.20.10

ip dhcp excluded-address 192.168.30.1  192.168.30.10

ip dhcp excluded-address 192.168.40.1  192.168.40.10

ip dhcp excluded-address 192.168.99.1  192.168.99.20

ip dhcp pool VLAN10-DEPTO1

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool VLAN20-DEPTO2

network 192.168.20.0 255.255.255.128

default-router 192.168.20.1

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool VLAN30-DEPTO3

network 192.168.30.0 255.255.255.128

default-router 192.168.30.1

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool VLAN40-DEPTO4

network 192.168.40.0 255.255.255.128

default-router 192.168.40.1

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

OSPF ---

router ospf 763

router-id 11.11.11.1

passive-interface vlan 10

passive-interface vlan 20

passive-interface vlan 30

passive-interface vlan 40

passive-interface vlan 99

STP ---

spanning-tree mode rapid-pvst

spanning-tree vlan 10,30,99 root primary

spanning-tree vlan 20,40    root secondary

DHCP SNOOPING / DAI ---

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,40,99

no ip dhcp snooping information option

ip arp inspection vlan 10,20,30,40,99

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

end

## 4.4  R-SW2 — Multilayer Switch Distribución SD

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Catalyst Multilayer Switch (L3) |
| Ubicación | Sede Santo Domingo — Capa Distribución |
| Descripción | Switch de distribución secundario en SD. HSRP Activo para VLANs 20, 40 y Standby para VLANs 10, 30. Configuración mirror de R-SW1 para redundancia completa de capa 3. |

Configuración completa:

hostname R-SW2

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

ip routing

VLANs (igual que R-SW1) ---

vlan 10

name DEPTO1

vlan 20

name DEPTO2

vlan 30

name DEPTO3

vlan 40

name DEPTO4

vlan 99

name MANAGEMENT

vlan 999

name NATIVA

UPLINK A R-CORE ---

interface e0/0

no switchport

ip address 192.168.100.6 255.255.255.252

ip ospf 763 area 1

no shutdown

TRUNK A R-SW1 (redundancia) ---

interface e0/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

no shutdown

TRUNK A SW3 (VLAN 30) ---

interface e0/1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 30,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

no shutdown

TRUNK A SW4 (VLAN 40) ---

interface e0/2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 40,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

no shutdown

SVIs CON HSRP (invertido respecto a R-SW1) ---

interface vlan 10

ip address 192.168.10.3 255.255.255.0

standby version 2

standby 10 ip 192.168.10.1

standby 10 priority 90

standby 10 preempt delay minimum 30

ip ospf 763 area 1

no shutdown

interface vlan 20

ip address 192.168.20.3 255.255.255.128

standby version 2

standby 20 ip 192.168.20.1

standby 20 priority 110

standby 20 preempt delay minimum 30

ip ospf 763 area 1

no shutdown

interface vlan 30

ip address 192.168.30.3 255.255.255.128

standby version 2

standby 30 ip 192.168.30.1

standby 30 priority 90

standby 30 preempt delay minimum 30

ip ospf 763 area 1

no shutdown

interface vlan 40

ip address 192.168.40.3 255.255.255.128

standby version 2

standby 40 ip 192.168.40.1

standby 40 priority 110

standby 40 preempt delay minimum 30

ip ospf 763 area 1

no shutdown

interface vlan 99

ip address 192.168.99.3 255.255.255.0

standby version 2

standby 99 ip 192.168.99.1

standby 99 priority 90

standby 99 preempt delay minimum 30

ip ospf 763 area 1

no shutdown

OSPF / STP / DHCP SNOOPING / DAI ---

router ospf 763

router-id 22.22.22.1

passive-interface vlan 10

passive-interface vlan 20

passive-interface vlan 30

passive-interface vlan 40

passive-interface vlan 99

spanning-tree mode rapid-pvst

spanning-tree vlan 20,40    root primary

spanning-tree vlan 10,30,99 root secondary

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,40,99

no ip dhcp snooping information option

ip arp inspection vlan 10,20,30,40,99

(DHCP pools: mismo patrón que R-SW1)

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.5  SW1–SW4 — Access Switches Sede Santo Domingo

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Catalyst Access Switch (L2) |
| Ubicación | Sede Santo Domingo — Capa Acceso |
| Descripción | Cuatro switches de acceso, uno por departamento. SW1 maneja VLAN 10 (Depto 1), SW2 → VLAN 20, SW3 → VLAN 30, SW4 → VLAN 40. Todos implementan seguridad de capa 2: Port Security, BPDU Guard, DHCP Snooping, DAI y Storm Control. Se muestra la configuración de SW1; los demás siguen el mismo patrón ajustando la VLAN y la IP de gestión. |

Configuración completa:

hostname SW1

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

VLANs ---

vlan 10

name DEPTO1

vlan 99

name MANAGEMENT

vlan 999

name NATIVA

TRUNK PRINCIPAL → R-SW1 ---

interface e0/0

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

spanning-tree guard root

no shutdown

TRUNK REDUNDANTE → R-SW2 ---

interface e0/1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

spanning-tree guard root

no shutdown

PUERTOS DE ACCESO DEPTO 1 ---

interface range e0/2-3, e1/0-3

switchport mode access

switchport access vlan 10

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

ip dhcp snooping limit rate 15

ip arp inspection limit rate 100

storm-control broadcast level 20.00

storm-control action shutdown

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security aging time 10

no shutdown

PUERTOS NO USADOS ---

interface range e2/0-3

description "UNUSED"

shutdown

switchport mode access

switchport access vlan 999

MANAGEMENT ---

interface vlan 99

ip address 192.168.99.11 255.255.255.0

no shutdown

ip default-gateway 192.168.99.1

DHCP SNOOPING / DAI / STP ---

ip dhcp snooping

ip dhcp snooping vlan 10,99

no ip dhcp snooping information option

ip arp inspection vlan 10,99

spanning-tree mode rapid-pvst

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

Cambios para cada switch adicional:

SW2: hostname SW2 | vlan 20 DEPTO2 | trunk allowed 20,99,999

vlan 99 ip 192.168.99.12 | uplink principal → R-SW1

SW3: hostname SW3 | vlan 30 DEPTO3 | trunk allowed 30,99,999

vlan 99 ip 192.168.99.13 | uplink principal → R-SW2

SW4: hostname SW4 | vlan 40 DEPTO4 | trunk allowed 40,99,999

vlan 99 ip 192.168.99.14 | uplink principal → R-SW2

## 4.6  R-SANTIAGO — Router Sede Santiago

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Router Cisco (con soporte IPSec/GRE) |
| Ubicación | Sede Santiago — Gateway y ABR OSPF |
| Descripción | Router de la sede Santiago. Gateway hacia Internet, ABR OSPF entre Área 0 (VPN) y Área 2 (LAN STG). NAT overload para salida a Internet. Subinterfaces 802.1Q hacia SW-STGO1. ip helper-address para DHCP remoto en VLANs 20 y 30. |

Configuración completa:

hostname R-SANTIAGO

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

ip routing

ISAKMP / IPSEC ---

crypto isakmp policy 10

encr aes 256

hash sha256

authentication pre-share

group 14

lifetime 86400

crypto isakmp key Moratti@2025 address 19.0.0.1

crypto ipsec transform-set TSET esp-aes 256 esp-sha256-hmac

mode transport

crypto ipsec profile VPN-PROFILE

set transform-set TSET

INTERFACES ---

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface e0/0

description "WAN — Enlace a FSP"

ip address 19.0.0.6 255.255.255.252

ip nat outside

ip access-group PROTECT-WAN-STG in

no shutdown

interface e0/1

description "LAN trunk — Switches Santiago"

no shutdown

interface e0/1.10

description "VLAN 10 — Centro de Datos"

encapsulation dot1Q 10

ip address 192.168.50.1 255.255.255.240

ip nat inside

ip ospf 763 area 2

no shutdown

interface e0/1.20

description "VLAN 20 — Ventas"

encapsulation dot1Q 20

ip address 192.168.50.33 255.255.255.224

ip nat inside

ip ospf 763 area 2

ip helper-address 192.168.50.2

no shutdown

interface e0/1.30

description "VLAN 30 — Administracion"

encapsulation dot1Q 30

ip address 192.168.50.65 255.255.255.240

ip nat inside

ip ospf 763 area 2

ip helper-address 192.168.50.2

no shutdown

interface e0/1.99

description "VLAN 99 — Management"

encapsulation dot1Q 99

ip address 192.168.99.20 255.255.255.0

no shutdown

interface e0/1.999

encapsulation dot1Q 999 native

no shutdown

TUNEL GRE/IPSEC hacia R-CORE ---

interface Tunnel0

description "VPN-GRE → R-CORE"

ip address 192.168.200.2 255.255.255.252

ip ospf 763 area 0

tunnel source e0/0

tunnel destination 19.0.0.1

tunnel protection ipsec profile VPN-PROFILE

no shutdown

OSPF ---

router ospf 763

router-id 2.2.2.2

passive-interface e0/0

passive-interface e0/1.10

passive-interface e0/1.20

passive-interface e0/1.30

passive-interface e0/1.99

NAT OVERLOAD ---

ip access-list extended NAT-ACL-STG

deny   ip 192.168.0.0 0.0.255.255 192.168.0.0 0.0.255.255

permit ip 192.168.50.0 0.0.0.127  any

ip nat inside source list NAT-ACL-STG interface e0/0 overload

ip route 0.0.0.0 0.0.0.0 19.0.0.5

ACL WAN ---

ip access-list extended PROTECT-WAN-STG

deny   tcp any any eq 23

permit ip any any

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.7  SW-STGO1 — Access Switch Sede Santiago

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Catalyst Access Switch (L2) |
| Ubicación | Sede Santiago — Capa Acceso |
| Descripción | Switch de acceso único en Santiago. Conecta servidores del Centro de Datos (VLAN 10), hosts de Ventas (VLAN 20) y Administración (VLAN 30). Port Security estricto en puertos de servidores (máx. 1 MAC). |

Configuración completa:

hostname SW-STGO1

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

VLANs ---

vlan 10

name CENTRO-DATOS

vlan 20

name VENTAS

vlan 30

name ADMINISTRACION

vlan 99

name MANAGEMENT

vlan 999

name NATIVA

TRUNK UPLINK → R-SANTIAGO ---

interface e0/0

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

spanning-tree guard root

no shutdown

PUERTOS SERVIDORES (VLAN 10) ---

e0/1: DNS/WEB/DHCP (.2) | e0/2: FTP/RADIUS (.3) | e0/3: MAIL (.4)

interface range e0/1-3

switchport mode access

switchport access vlan 10

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security maximum 1

switchport port-security violation shutdown

no shutdown

PUERTOS VENTAS (VLAN 20) ---

interface range e1/0-3

switchport mode access

switchport access vlan 20

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

ip dhcp snooping limit rate 15

ip arp inspection limit rate 100

switchport port-security maximum 2

switchport port-security violation shutdown

no shutdown

PUERTOS ADMINISTRACION (VLAN 30) ---

interface range e2/0-2

switchport mode access

switchport access vlan 30

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security maximum 2

switchport port-security violation shutdown

no shutdown

PUERTOS NO USADOS ---

interface range e2/3, e3/0-3

description "UNUSED"

shutdown

switchport mode access

switchport access vlan 999

MANAGEMENT ---

interface vlan 99

ip address 192.168.99.21 255.255.255.0

no shutdown

ip default-gateway 192.168.99.20

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,99

no ip dhcp snooping information option

ip arp inspection vlan 10,20,30,99

spanning-tree mode rapid-pvst

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.8  R-ROMANA — Router Sede La Romana

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Router Cisco (con soporte IPSec/GRE) |
| Ubicación | Sede La Romana — Gateway y ABR OSPF |
| Descripción | Router de la sede La Romana. ABR OSPF entre Área 0 (VPN) y Área 3. NAT overload para Internet. DHCP local para las tres VLANs. ACL de seguridad que restringe el Depto 3 a solo acceder al Depto 1 (impresión). |

Configuración completa:

hostname R-ROMANA

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

ip routing

ISAKMP / IPSEC ---

crypto isakmp policy 10

encr aes 256

hash sha256

authentication pre-share

group 14

lifetime 86400

crypto isakmp key Moratti@2025 address 19.0.0.1

crypto ipsec transform-set TSET esp-aes 256 esp-sha256-hmac

mode transport

crypto ipsec profile VPN-PROFILE

set transform-set TSET

INTERFACES ---

interface Loopback0

ip address 3.3.3.3 255.255.255.255

interface e0/0

description "WAN — Enlace a FSP"

ip address 19.0.0.10 255.255.255.252

ip nat outside

ip access-group PROTECT-WAN-ROM in

no shutdown

interface e0/1

description "LAN trunk — Switch La Romana"

no shutdown

interface e0/1.10

description "VLAN 10 — Depto 1"

encapsulation dot1Q 10

ip address 192.168.60.1 255.255.255.192

ip nat inside

ip ospf 763 area 3

no shutdown

interface e0/1.20

description "VLAN 20 — Depto 2"

encapsulation dot1Q 20

ip address 192.168.60.129 255.255.255.128

ip nat inside

ip ospf 763 area 3

no shutdown

interface e0/1.30

description "VLAN 30 — Depto 3"

encapsulation dot1Q 30

ip address 192.168.60.65 255.255.255.240

ip nat inside

ip ospf 763 area 3

ip access-group ACL-ROMANA-SEGURIDAD in

no shutdown

interface e0/1.99

encapsulation dot1Q 99

ip address 192.168.99.30 255.255.255.0

no shutdown

interface e0/1.999

encapsulation dot1Q 999 native

no shutdown

TUNEL GRE/IPSEC hacia R-CORE ---

interface Tunnel0

description "VPN-GRE → R-CORE"

ip address 192.168.200.6 255.255.255.252

ip ospf 763 area 0

tunnel source e0/0

tunnel destination 19.0.0.1

tunnel protection ipsec profile VPN-PROFILE

no shutdown

DHCP POOLS ---

ip dhcp excluded-address 192.168.60.1  192.168.60.5

ip dhcp excluded-address 192.168.60.65 192.168.60.70

ip dhcp excluded-address 192.168.60.129 192.168.60.135

ip dhcp pool ROMANA-VLAN10-DEPTO1

network 192.168.60.0 255.255.255.192

default-router 192.168.60.1

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool ROMANA-VLAN20-DEPTO2

network 192.168.60.128 255.255.255.128

default-router 192.168.60.129

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool ROMANA-VLAN30-DEPTO3

network 192.168.60.64 255.255.255.240

default-router 192.168.60.65

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

OSPF ---

router ospf 763

router-id 3.3.3.3

passive-interface e0/0

passive-interface e0/1.10

passive-interface e0/1.20

passive-interface e0/1.30

passive-interface e0/1.99

NAT OVERLOAD ---

ip access-list extended NAT-ACL-ROM

deny   ip 192.168.0.0 0.0.255.255 192.168.0.0 0.0.255.255

permit ip 192.168.60.0 0.0.0.255  any

ip nat inside source list NAT-ACL-ROM interface e0/0 overload

ACL SEGURIDAD INTERDEPT (Depto3 solo accede a Depto1) ---

ip access-list extended ACL-ROMANA-SEGURIDAD

permit ip 192.168.60.64 0.0.0.15 192.168.60.0 0.0.0.63

deny   ip 192.168.60.64 0.0.0.15 192.168.60.0 0.0.0.255

deny   tcp any any eq 23

permit ip any any

ip route 0.0.0.0 0.0.0.0 19.0.0.9

ip access-list extended PROTECT-WAN-ROM

deny   tcp any any eq 23

permit ip any any

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.10  R-PUERTOPLATA — Router Sede Puerto Plata

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Router Cisco (con soporte IPSec/GRE) |
| Ubicación | Sede Puerto Plata — Gateway y ABR OSPF |
| Descripción | Router de la sede Puerto Plata. ABR OSPF entre Área 0 (VPN) y Área 4. NAT overload para Internet. DHCP local para las tres VLANs con crecimiento ×1.4 aplicado. |

Configuración completa:

hostname R-PUERTOPLATA

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

ip routing

username admin privilege 15 secret Admin@ITLA2025

ip domain-name moratti.com.do

crypto key generate rsa modulus 2048

ip ssh version 2

crypto isakmp policy 10

encr aes 256

hash sha256

authentication pre-share

group 14

lifetime 86400

crypto isakmp key Moratti@2025 address 19.0.0.1

crypto ipsec transform-set TSET esp-aes 256 esp-sha256-hmac

mode transport

crypto ipsec profile VPN-PROFILE

set transform-set TSET

interface Loopback0

description "Router-ID OSPF"

ip address 4.4.4.4 255.255.255.255

interface e0/0

description "WAN — Enlace a FSP"

ip address 19.0.0.14 255.255.255.252

ip nat outside

ip access-group ACL-PP-SEGURIDAD in

no shutdown

interface e0/1

description "LAN trunk — Switch Puerto Plata"

no shutdown

interface e0/1.10

description "VLAN 10 — Depto 1 (25h->35h con crecimiento)"

encapsulation dot1Q 10

ip address 192.168.70.1 255.255.255.192

ip nat inside

ip ospf 763 area 4

no shutdown

interface e0/1.20

description "VLAN 20 — Depto 2 (52h->73h con crecimiento)"

encapsulation dot1Q 20

ip address 192.168.70.65 255.255.255.128

ip nat inside

ip ospf 763 area 4

no shutdown

interface e0/1.30

description "VLAN 30 — Depto 3 (7h->10h con crecimiento)"

encapsulation dot1Q 30

ip address 192.168.70.193 255.255.255.240

ip nat inside

ip ospf 763 area 4

no shutdown

interface e0/1.99

description "VLAN 99 — Management"

encapsulation dot1Q 99

ip address 192.168.99.40 255.255.255.0

no shutdown

interface e0/1.999

encapsulation dot1Q 999 native

no shutdown

interface Tunnel0

description "VPN-GRE -> R-CORE"

ip address 192.168.200.10 255.255.255.252

ip ospf 763 area 0

tunnel source e0/0

tunnel destination 19.0.0.1

tunnel protection ipsec profile VPN-PROFILE

no shutdown

ip dhcp excluded-address 192.168.70.1   192.168.70.5

ip dhcp excluded-address 192.168.70.65  192.168.70.70

ip dhcp excluded-address 192.168.70.193 192.168.70.196

ip dhcp pool PP-VLAN10-DEPTO1

network 192.168.70.0 255.255.255.192

default-router 192.168.70.1

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool PP-VLAN20-DEPTO2

network 192.168.70.64 255.255.255.128

default-router 192.168.70.65

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool PP-VLAN30-DEPTO3

network 192.168.70.192 255.255.255.240

default-router 192.168.70.193

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

router ospf 763

router-id 4.4.4.4

passive-interface e0/0

passive-interface e0/1.10

passive-interface e0/1.20

passive-interface e0/1.30

passive-interface e0/1.99

ip access-list extended NAT-ACL-PP

deny   ip 192.168.0.0 0.0.255.255 192.168.0.0 0.0.255.255

permit ip 192.168.70.0 0.0.0.255  any

ip nat inside source list NAT-ACL-PP interface e0/0 overload

ip access-list extended ACL-PP-SEGURIDAD

deny   tcp any any eq 23

deny   tcp any any eq 513

permit ip any any

ip route 0.0.0.0 0.0.0.0 19.0.0.13

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.11  SW-PP — Access Switch Sede Puerto Plata

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Catalyst Access Switch (L2) |
| Ubicación | Sede Puerto Plata — Capa Acceso |
| Descripción | Switch de acceso en Puerto Plata. Conecta los tres departamentos de la sede. Implementa seguridad de capa 2: Port Security, BPDU Guard, DHCP Snooping, DAI y Storm Control. |

Configuración completa:

hostname SW-PP

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

username admin privilege 15 secret Admin@ITLA2025

ip domain-name moratti.com.do

crypto key generate rsa modulus 2048

ip ssh version 2

vlan 10

name DEPTO1-PP

vlan 20

name DEPTO2-PP

vlan 30

name DEPTO3-PP

vlan 99

name MANAGEMENT

vlan 999

name NATIVA

interface e0/0

description "Trunk -> R-PUERTOPLATA"

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

spanning-tree guard root

no shutdown

interface range e0/1-3, e1/0-1

description "Hosts Depto 1 — VLAN 10"

switchport mode access

switchport access vlan 10

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

ip dhcp snooping limit rate 15

ip arp inspection limit rate 100

storm-control broadcast level 20.00

storm-control action shutdown

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security aging time 10

no shutdown

interface range e1/2-3, e2/0-3, e3/0-3

description "Hosts Depto 2 — VLAN 20"

switchport mode access

switchport access vlan 20

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

ip dhcp snooping limit rate 15

ip arp inspection limit rate 100

storm-control broadcast level 20.00

storm-control action shutdown

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security aging time 10

no shutdown

interface vlan 99

ip address 192.168.99.41 255.255.255.0

no shutdown

ip default-gateway 192.168.99.40

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,99

no ip dhcp snooping information option

ip arp inspection vlan 10,20,30,99

spanning-tree mode rapid-pvst

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.12  R-BARAHONA — Router Sede Barahona

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Router Cisco (con soporte IPSec/GRE) |
| Ubicación | Sede Barahona — Gateway y ABR OSPF |
| Descripción | Router de la sede Barahona. ABR OSPF entre Área 0 (VPN) y Área 5. NAT overload para Internet. DHCP local para las tres VLANs con crecimiento ×1.4 aplicado. |

Configuración completa:

hostname R-BARAHONA

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

ip routing

username admin privilege 15 secret Admin@ITLA2025

ip domain-name moratti.com.do

crypto key generate rsa modulus 2048

ip ssh version 2

crypto isakmp policy 10

encr aes 256

hash sha256

authentication pre-share

group 14

lifetime 86400

crypto isakmp key Moratti@2025 address 19.0.0.1

crypto ipsec transform-set TSET esp-aes 256 esp-sha256-hmac

mode transport

crypto ipsec profile VPN-PROFILE

set transform-set TSET

interface Loopback0

description "Router-ID OSPF"

ip address 5.5.5.5 255.255.255.255

interface e0/0

description "WAN — Enlace a FSP"

ip address 19.0.0.18 255.255.255.252

ip nat outside

ip access-group ACL-BAR-SEGURIDAD in

no shutdown

interface e0/1

description "LAN trunk — Switch Barahona"

no shutdown

interface e0/1.10

description "VLAN 10 — Depto 1 (25h->35h con crecimiento)"

encapsulation dot1Q 10

ip address 192.168.80.1 255.255.255.192

ip nat inside

ip ospf 763 area 5

no shutdown

interface e0/1.20

description "VLAN 20 — Depto 2 (52h->73h con crecimiento)"

encapsulation dot1Q 20

ip address 192.168.80.65 255.255.255.128

ip nat inside

ip ospf 763 area 5

no shutdown

interface e0/1.30

description "VLAN 30 — Depto 3 (7h->10h con crecimiento)"

encapsulation dot1Q 30

ip address 192.168.80.193 255.255.255.240

ip nat inside

ip ospf 763 area 5

no shutdown

interface e0/1.99

description "VLAN 99 — Management"

encapsulation dot1Q 99

ip address 192.168.99.50 255.255.255.0

no shutdown

interface e0/1.999

encapsulation dot1Q 999 native

no shutdown

interface Tunnel0

description "VPN-GRE -> R-CORE"

ip address 192.168.200.14 255.255.255.252

ip ospf 763 area 0

tunnel source e0/0

tunnel destination 19.0.0.1

tunnel protection ipsec profile VPN-PROFILE

no shutdown

ip dhcp excluded-address 192.168.80.1   192.168.80.5

ip dhcp excluded-address 192.168.80.65  192.168.80.70

ip dhcp excluded-address 192.168.80.193 192.168.80.196

ip dhcp pool BAR-VLAN10-DEPTO1

network 192.168.80.0 255.255.255.192

default-router 192.168.80.1

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool BAR-VLAN20-DEPTO2

network 192.168.80.64 255.255.255.128

default-router 192.168.80.65

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

ip dhcp pool BAR-VLAN30-DEPTO3

network 192.168.80.192 255.255.255.240

default-router 192.168.80.193

dns-server 192.168.50.2

domain-name moratti.com.do

lease 1

router ospf 763

router-id 5.5.5.5

passive-interface e0/0

passive-interface e0/1.10

passive-interface e0/1.20

passive-interface e0/1.30

passive-interface e0/1.99

ip access-list extended NAT-ACL-BAR

deny   ip 192.168.0.0 0.0.255.255 192.168.0.0 0.0.255.255

permit ip 192.168.80.0 0.0.0.255  any

ip nat inside source list NAT-ACL-BAR interface e0/0 overload

ip access-list extended ACL-BAR-SEGURIDAD

deny   tcp any any eq 23

deny   tcp any any eq 513

permit ip any any

ip route 0.0.0.0 0.0.0.0 19.0.0.17

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.13  SW-BAR — Access Switch Sede Barahona

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Catalyst Access Switch (L2) |
| Ubicación | Sede Barahona — Capa Acceso |
| Descripción | Switch de acceso en Barahona. Misma estructura que SW-PP ajustando VLANs y la red 192.168.80.x. Incluye toda la seguridad de capa 2 estándar de la empresa. |

Configuración completa:

hostname SW-BAR

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

username admin privilege 15 secret Admin@ITLA2025

ip domain-name moratti.com.do

crypto key generate rsa modulus 2048

ip ssh version 2

vlan 10

name DEPTO1-BAR

vlan 20

name DEPTO2-BAR

vlan 30

name DEPTO3-BAR

vlan 99

name MANAGEMENT

vlan 999

name NATIVA

interface e0/0

description "Trunk -> R-BARAHONA"

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

spanning-tree guard root

no shutdown

interface range e0/1-3, e1/0-1

description "Hosts Depto 1 — VLAN 10"

switchport mode access

switchport access vlan 10

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

ip dhcp snooping limit rate 15

ip arp inspection limit rate 100

storm-control broadcast level 20.00

storm-control action shutdown

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security aging time 10

no shutdown

interface range e1/2-3, e2/0-3, e3/0-3

description "Hosts Depto 2 — VLAN 20"

switchport mode access

switchport access vlan 20

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

ip dhcp snooping limit rate 15

ip arp inspection limit rate 100

storm-control broadcast level 20.00

storm-control action shutdown

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security aging time 10

no shutdown

interface vlan 99

ip address 192.168.99.51 255.255.255.0

no shutdown

ip default-gateway 192.168.99.50

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,99

no ip dhcp snooping information option

ip arp inspection vlan 10,20,30,99

spanning-tree mode rapid-pvst

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

## 4.9  SW-ROM — Access Switch Sede La Romana

| Atributo | Valor |
| --- | --- |
| Tipo de equipo | Catalyst Access Switch (L2) |
| Ubicación | Sede La Romana — Capa Acceso |
| Descripción | Switch de acceso en La Romana. Conecta los tres departamentos de la sede remota. Implementa el mismo conjunto de seguridad de capa 2 que los switches de SD. |

Configuración completa:

hostname SW-ROM

enable secret MORATTI2025

no ip domain-lookup

service password-encryption

VLANs ---

vlan 10

name DEPTO1-ROM

vlan 20

name DEPTO2-ROM

vlan 30

name DEPTO3-ROM

vlan 99

name MANAGEMENT

vlan 999

name NATIVA

TRUNK UPLINK → R-ROMANA ---

interface e0/0

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,99,999

switchport nonegotiate

ip dhcp snooping trust

ip arp inspection trust

spanning-tree guard root

no shutdown

PUERTOS ACCESO VLAN 10 (DEPTO 1 — 25 hosts) ---

interface range e0/1-3, e1/0-2

switchport mode access

switchport access vlan 10

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

ip dhcp snooping limit rate 15

ip arp inspection limit rate 100

storm-control broadcast level 20.00

storm-control action shutdown

switchport port-security maximum 2

switchport port-security violation shutdown

no shutdown

PUERTOS ACCESO VLAN 20 (DEPTO 2 — 52 hosts) ---

interface range e1/3, e2/0-3, e3/0-3

switchport mode access

switchport access vlan 20

switchport nonegotiate

spanning-tree portfast

spanning-tree bpduguard enable

ip dhcp snooping limit rate 15

ip arp inspection limit rate 100

storm-control broadcast level 20.00

storm-control action shutdown

switchport port-security maximum 2

switchport port-security violation shutdown

no shutdown

MANAGEMENT ---

interface vlan 99

ip address 192.168.99.31 255.255.255.0

no shutdown

ip default-gateway 192.168.99.30

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,99

no ip dhcp snooping information option

ip arp inspection vlan 10,20,30,99

spanning-tree mode rapid-pvst

ip access-list standard SSH-MGMT

permit 192.168.99.0 0.0.0.255

deny   any

line console 0

login local

logging synchronous

line vty 0 4

login local

transport input ssh

access-class SSH-MGMT in

end

# 5. Descripción de Equipos y Justificación

A continuación se detalla cada tipo de equipo utilizado en el proyecto, con su función técnica y la justificación de su selección dentro del diseño de red.

| Equipo | Modelo referencia | Función | Justificación |
| --- | --- | --- | --- |
| Router WAN (R-CORE, R-SANTIAGO, R-ROMANA, R-PUERTOPLATA, R-BARAHONA) | Cisco ISR 4321 / GNS3 IOSv | Gateway WAN, VPN Hub/Spoke, ABR OSPF, NAT overload | Soporte nativo de IPSec, GRE, OSPF multi-área y NAT. Suficiente throughput para las 5 sucursales. |
| FSP (Router ISP simulado) | Cisco IOSv / Eve-NG | Simula la nube/Internet y los enlaces WAN públicos | Necesario en laboratorio para emular el ISP sin depender de conectividad real. |
| Multilayer Switch Distribución (R-SW1, R-SW2) | Cisco Catalyst 3560 / 3750 | Enrutamiento inter-VLAN, HSRP, DHCP, STP Root, DAI | Capacidad de enrutamiento L3, soporte de SVIs y HSRP para alta disponibilidad sin router adicional. |
| Access Switch Sede SD (SW1–SW4) | Cisco Catalyst 2960 | Segmentación VLAN, Port Security, BPDU Guard, DHCP Snooping | Economico y confiable para acceso de usuario final. Soporte completo de seguridad L2. |
| Access Switch Santiago (SW-STGO1) | Cisco Catalyst 2960 | Conectar servidores y hosts en Santiago | Mismo modelo que SD para estandarización y reducción de costos de mantenimiento. |
| Access Switch La Romana (SW-ROM) | Cisco Catalyst 2960 | Conectar hosts de los tres departamentos en ROM | Idem SD y Santiago. Configuración simplificada al ser sucursal de menor tamaño. |
| Access Switch Puerto Plata (SW-PP) | Cisco Catalyst 2960 | Conectar hosts de los tres departamentos en Puerto Plata | Mismo modelo estándar de la empresa. DHCP local desde R-PUERTOPLATA. |
| Access Switch Barahona (SW-BAR) | Cisco Catalyst 2960 | Conectar hosts de los tres departamentos en Barahona | Mismo modelo estándar. Configuración idéntica a SW-PP ajustando VLANs y red 192.168.80.x. |
| Servidores (DNS/DHCP/WEB/FTP/MAIL) | Servidor x86 / VM en Data Center STG | Servicios centralizados de infraestructura para toda la empresa | Centralizar los servicios en Santiago reduce costos y simplifica la administración. El DHCP centralizado usa ip helper-address. |

## 5.1 Justificación de Tecnologías Implementadas

| Tecnología | Por qué se eligió |
| --- | --- |
| OSPF Multi-área | Protocolo de enrutamiento de estado de enlace escalable. Las áreas reducen el tamaño de la base de datos LSDB y el tráfico de actualizaciones. El Area 0 actúa como backbone obligatorio según RFC 2328. |
| GRE sobre IPSec | GRE permite encapsular tráfico multicast/broadcast (necesario para OSPF) dentro del túnel. IPSec provee confidencialidad (AES-256) e integridad (SHA-256). La combinación resuelve la limitación de IPSec puro con OSPF. |
| HSRP (Hot Standby Router Protocol) | Proporciona redundancia de gateway de primer salto (FHRP) sin impacto en los hosts. Si R-SW1 falla, R-SW2 asume automáticamente la IP virtual sin cambiar la configuración de los clientes. |
| NAT Overload (PAT) | Permite que toda la red privada 192.168.0.0/16 salga a Internet usando una sola IP pública por sede. La ACL extendida garantiza que el tráfico inter-sede viaje por el túnel VPN, no por NAT. |
| VLAN 99 — Management | Separar el tráfico de gestión del tráfico de datos es una buena práctica de seguridad (principio de menor privilegio). El acceso SSH está restringido exclusivamente a esta VLAN. |
| DHCP Snooping + DAI | DHCP Snooping previene servidores DHCP rogue. DAI (Dynamic ARP Inspection) usa la tabla de DHCP Snooping para validar respuestas ARP, mitigando ataques de envenenamiento ARP (ARP Spoofing). |
| Rapid-PVST+ (STP) | Convergencia rápida por VLAN (< 1 s) frente a STP clásico (30-50 s). PortFast + BPDU Guard en puertos de acceso protegen contra topología loops accidentales o intentos de manipulación. |
| Port Security | Limita el número de MACs aprendidas por puerto (máx. 2 hosts). Ante una violación el puerto se apaga automáticamente (shutdown), mitigando ataques de MAC flooding. |

# 6. Comandos de Verificación

Los siguientes comandos deben ejecutarse en cada dispositivo para validar el correcto funcionamiento de todos los protocolos y funcionalidades implementadas.

## 6.1 OSPF

show ip ospf neighbor               ! Ver vecinos OSPF y estado

show ip ospf database               ! Ver base de datos LSDB

show ip route ospf                  ! Ver rutas aprendidas por OSPF

show ip ospf interface brief        ! Estado de interfaces OSPF

## 6.2 HSRP (en R-SW1 y R-SW2)

show standby brief                  ! Estado resumido de todos los grupos

show standby vlan 10                ! Detalle grupo VLAN 10

show standby vlan 20                ! Detalle grupo VLAN 20

## 6.3 VPN / IPSec

show crypto isakmp sa               ! Ver SAs de IKE Fase 1

show crypto ipsec sa                ! Ver SAs de IPSec Fase 2

show interfaces Tunnel0             ! Estado del túnel GRE

show interfaces Tunnel1             ! (solo en R-CORE)

## 6.4 DHCP

show ip dhcp binding                ! Ver asignaciones activas

show ip dhcp pool                   ! Ver configuración de los pools

show ip dhcp conflict               ! Ver conflictos de IP

## 6.5 NAT

show ip nat translations            ! Ver traducciones activas

show ip nat statistics              ! Estadísticas de NAT

## 6.6 Seguridad Capa 2

show ip dhcp snooping binding       ! Tabla de DHCP Snooping

show ip arp inspection statistics   ! Estadísticas de DAI

show port-security interface <int>  ! Estado de port-security

show spanning-tree vlan <ID>        ! Árbol STP por VLAN

## 6.7 Pruebas de Conectividad

ping 192.168.200.2 source Tunnel0   ! R-CORE → R-SANTIAGO (VPN)

ping 192.168.200.6 source Tunnel0   ! R-CORE → R-ROMANA (VPN)

ping 192.168.50.1 source 192.168.10.1  ! SD → Santiago (inter-sede)

ping 192.168.60.1 source 192.168.10.1  ! SD → La Romana (inter-sede)

ping 192.168.70.1 source 192.168.10.1  ! SD → Puerto Plata (inter-sede)

ping 192.168.80.1 source 192.168.10.1  ! SD → Barahona (inter-sede)

ping 8.8.8.8 source 192.168.10.1    ! SD → Internet (NAT)